#### General Notes for Post-Installed Rebar

POST-INSTALLED REINFORCING BAR CONNECTIONS SHALL BE DESIGNED PER THE CSA A23.3-14 CODE REQUIREMENTS FOR STRUCTURAL CONCRETE. POST-INSTALLED REINFORCING BAR CONNECTIONS SHALL CONSIST OF THE FOLLOWING ADHESIVE SYSTEMS AS PROVIDED BY HILTI (CANADA) CORPORATION.
CONTACT HILTI AT (800) 363-4458 FOR PRODUCT RELATED QUESTIONS.

- POST-INSTALLED REINFORCING BAR CONNECTIONS IN CONCRETE

  - ADHESIVE SYSTEMS WITH SafeSet™ TECHNOLOGY:

    - HILTI HIT-RE 500 V3 SAFESET SYSTEM WITH HILTI HOLLOW DRILL BIT (TE-CD OR TE-YD), AND HILTI VC 150/300 VACUUM (VC 150 OR VC 300) OR OTHER APPROVED DRILLING METHOD WITH POST-INSTALLED STEEL REINFORCING BARS

    - HILTI HIT-HY 200 SAFESET SYSTEM WITH HILTI HOLLOW DRILL BIT (TE-CD OR TE-YD), AND HILTI VC 150/300 VACUUM (VC 150 OR VC 300) OR OTHER APPROVED DRILLING METHOD WITH POST-INSTALLED STEEL REINFORCING BARS

- THE DESIGN OF STRAIGHT POST-INSTALLED REINFORCING BARS SHALL BE PERFORMED PER THE DEVELOPMENT AND SPLICE REQUIREMENTS OF CSA A23.3-14 CODE REQUIREMENTS FOR STRUCTURAL CONCRETE. THE POST-INSTALLED REINFORCING BAR SYSTEM IS AN ALTERNATIVE TO CAST-IN-PLACE REINFORCING BARS GOVERNED BY CSA A23.3-14.

- THE ADHESIVE SYSTEM SHALL BE TESTED IN ACCORDANCE WITH THE ICC-ES ACCEPTANCE CRITERIA FOR POST-INSTALLED ADHESIVE ANCHORS IN CONCRETE ELEMENTS (AC308), TABLE 3.8. TECHNICAL DATA SHALL BE PUBLISHED IN AN ICC-ES EVALUATION SERVICE REPORT SHOWING COMPLIANCE WITH ACI 318-11 WHICH IS FUNCTIONALLY EQUIVALENT TO CSA A23.3-14

.

  - ICC-ES ESR-3814 FOR HILTI HIT-RE 500 V3 EPOXY ANCHORS AND POST-INSTALLED REINFORCING BAR CONNECTIONS IN CRACKED AND UNCRACKED CONCRETE

  - ICC-ES ESR-3187 FOR HILTI HIT HY-200 ADHESIVE ANCHORS AND POST-INSTALLED REINFORCING BAR CONNECTIONS IN CRACKED AND UNCRACKED CONCRETE.

- POST-INSTALLED REINFORCING BAR INSTALLATION SHALL BE PERFORMED BY PERSONNEL TRAINED TO INSTALL THE SYSTEM PER THE MANUFACTURER'S PRINTED INSTALLATION INSTRUCTIONS (MPII), AS INCLUDED IN THE ANCHOR PACKAGING. THE CONCTRACTOR SHALL ARRANGE FOR A MANUFACTURER’S REPRESENTATIVE TO PROVIDE ONSITE INSTALLATION TRAINING FOR POST-INSTALLED REINFORCING BARS. THE STRUCTURAL ENGINEER OF RECORD MUST RECEIVE DOCUMENTED CONFIRMATION PRIOR TO THE COMMENCEMENT OF INSTALLING THE BARS THAT ALL OF THE CONTRACTOR'S PERSONNEL WHO WILL INSTALL POST-INSTALLED REINFORCING BARS HAVE BEEN TRAINED TO INSTALL THE SYSTEM PER THE MANUFACTURER'S PRINTED INSTALLATION INSTRUCTIONS (MPII).

- THE POSITION OF EXISTING REINFORCING BARS IN THE CONCRETE STRUCTURE SHALL BE LOCATED PRIOR TO POST-INSTALLING BARS. EXISTING BARS SHALL BE LOCATED USING HILTI FERROSCAN, GPR, X-RAY, CHIPPING OR OTHER MEANS.